The Dana Point Condominium Association

Tuck pointing Specifications for leak repairs

Please include prices itemized by location and include any listed options separately.

Locations to be tuck pointed:

- 1605 Unit #420C – grind and tuckpoint top wall areas front of building 24ft. by 2-1/2ft, side wall 2-1/2ft. and 2-1/2ft. high top of 4th floor, replace bricks as needed, caulk coping  joints, and waterproof area.

  - Option to install 24 gauge steel coping over existing limestone coping.

- 1605 Unit #409 – front side only, rebuild top of lintel, rebuild outside brick top of window on 4th floor 24ft by 2-1/2 ft., rebuild inside brick to roof line, reuse limestone coping, install new base wall flashing, drip edge, and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1615 Unit #420C – Rebuild to top of lintel, rebuilding outside brick top of window on 4th floor 24ft by 2-1/2 ft., rebuild inside brick to roof, reuse limestone coping, install new base wall flashing, clean two lintels, install flashing, drip edge, and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1615 Unit #407A – Spot tuck point at two sides of unit, 32 ft. on East side and 30 ft. on South wall.

  - Option to install 24 gauge steel coping over existing limestone (70LNF)

- 1515 Unit #451/lobby edge – rebuild to top of lintel, rebuild outside  brick top of window on 4th floor 24ft by 2-1/2 ft., rebuilt inside brick to roof line, re-use limestone coping, install new base wall flashing, clean two lintels and install flashing, drip edge and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1515 Unit #459B - rebuild to top of lintel, rebuild outside  brick top of window on 4th floor 24ft by 2-1/2 ft., rebuilt inside brick to roof line, re-use limestone coping, install new base wall flashing, clean two lintels and install flashing, drip edge and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1515 Unit #462-464B – Spot tuck point where applicable.

  - Option to install 24 gauge steel coping over existing limestone

- 1515 Unit #466B North Side - rebuild to top of lintel, rebuild outside  brick top of window on 4th floor 24ft by 2-1/2 ft., rebuilt inside brick to roof line, re-use limestone coping, install new base wall flashing, clean two lintels and install flashing, drip edge and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1515 #466B East Side – Spot tuck point 30 ft. where needed.

  - Option to install 24 gauge steel coping over existing limestone

- 1515 #420C - rebuild to top of lintel, rebuild outside  brick top of window on 4th floor 24ft by 2-1/2 ft., rebuilt inside brick to roof line, re-use limestone coping, install new base wall flashing, clean two lintels and install flashing, drip edge and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 2505 #407A - Spot tuck point 30 ft. where needed.

  - Option to install 24 gauge steel coping over existing limestone

- 1505 #411 – 413B - rebuild to top of lintel, rebuild outside  brick top of window on 4th floor 24ft by 2-1/2 ft., rebuilt inside brick to roof line, re-use limestone coping, install new base wall flashing, clean two lintels and install flashing, drip edge and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1405 #407A - rebuild to top of lintel, rebuild outside  brick top of window on 4th floor 24ft by 2-1/2 ft., rebuilt inside brick to roof line, re-use limestone coping, install new base wall flashing, clean two lintels and install flashing, drip edge and termination bar.

  - Option to install 24 gauge steel coping over existing limestone

- 1605 #320c – Spot Tuck point and caulk as needed where  leaking into living room windows.

Proposal should be submitted to on-site Management Office no later than June 27 at 1519 E. Central Rd, Arlington Heights, IL  60005 or via email to LRay @LMSnet.com.